LEON3 + GRETH Ethernet Test

Complete Change Log

All Bugs Found and Fixes Applied — Session Record

| Field | Value |
| --- | --- |
| Target Design | GR-XC3S-1500 Template |
| Processor | LEON3 (SPARC V8) |
| Ethernet Core | GRETH (Gaisler Ethernet MAC) |
| Simulator | VCS (Synopsys) on Linux |
| Compiler | sparc-gaisler-elf-gcc |
| Reference Doc | GRETH_DETAILS.pdf (GRIP Oct 2025 v2025.2) |
| Total Bugs Fixed | 16 |

# 1. Overview

This document records every bug found and every fix applied to greth_test.c during the debugging session for the LEON3 + GRETH Ethernet verification test. Changes are grouped by category and presented in the order they were discovered. Each entry explains what was wrong, why it was wrong, and exactly what was changed.

The test has 5 sub-tests that exercise the full GRETH Ethernet MAC:

- Test 1 — GRETH register access and MAC address write-readback

- Test 2 — PHY MDIO read (PHY IDs and status)

- Test 3 — PHY loopback mode enable via MDIO write

- Test 4 — Single frame TX → PHY loopback → RX → payload verify

- Test 5 — Multi-frame stress loopback (4 frames)

# 2. GRETH Control Register Fixes

## 2.1  CTRL_PROM — Wrong Bit Position

| Field | Detail |
| --- | --- |
| Bug | CTRL_PROM was defined as (1<<8) = bit8 = RESERVED per GRETH datasheet |
| Effect | Promiscuous mode was never actually enabled. GRETH silently dropped every loopback frame because the broadcast destination MAC (FF:FF:FF:FF:FF:FF) did not match the programmed MAC address. This caused ALL RX failures in Tests 4 and 5. |
| Fix | Change define to (1<<5) per PDF Table 797: bit5 = PM (Promiscuous mode) |
| Code change | #define CTRL_PROM (1<<5) // was (1<<8) |

## 2.2  CTRL_100MB — Not Applicable in MII Mode

| Field | Detail |
| --- | --- |
| Bug | CTRL_100MB = (1<<7) was written to GRETH_CTRL and checked in Test 1 |
| Effect | PDF states: 'Speed (SP) bit7 — Only used in RMII mode (rmii=1). A default value is automatically read from the PHY after reset.' This design uses MII mode (rmii=0). Bit7 is read-only in MII mode and cannot be set by software. The '100Mbit bit set' check always failed. |
| Fix | Removed CTRL_100MB from all WR(GRETH_CTRL,...) calls. Removed the '100Mbit bit set' check from Test 1. Speed is determined automatically by GRETH from the PHY MII clock. |
| Code change | Deleted #define CTRL_100MB. Removed from all CTRL writes. |

## 2.3  CTRL_TXEN/CTRL_RXEN Written Without Descriptors in Test 1

| Field | Detail |
| --- | --- |
| Bug | Test 1 wrote CTRL_TXEN¦CTRL_RXEN as part of the enable check even though no TX or RX descriptors were set up. |
| Effect | PDF states: 'As soon as GRETH encounters a disabled descriptor it will stop until TE/RE is set again.' With no descriptors, GRETH found nothing enabled and immediately self-cleared TE and RE. CTRL values appeared to drop from 0x13 to 0x11 to 0x10 in the waveform — this looked like a bug but is correct hardware behavior. |
| Fix | Removed CTRL_TXEN and CTRL_RXEN from Test 1. Only CTRL_FULLD is written to verify bit4. TE and RE are only set in Tests 4 and 5 after descriptors are fully configured. |
| Code change | WR(GRETH_CTRL, CTRL_FULLD); // was CTRL_TXEN¦CTRL_RXEN¦CTRL_FULLD |

# 3. GRETH MDIO Register Fixes

The MDIO control/status register (offset 0x10) has a specific bit layout per PDF Table 801 that was misread in the original code. Multiple bits were wrong.

## 3.1  MDIO_BUSY — Wrong Bit Position

| Field | Detail |
| --- | --- |
| Bug | MDIO_BUSY was defined as (1<<0) = bit0 |
| Effect | PDF Table 801: 'Busy (BU) bit3'. Polling bit0 instead of bit3 meant mdio_wait() never actually waited for MDIO operations to complete. All MDIO reads returned garbage because the next operation started before the previous one finished. |
| Fix | #define MDIO_BUSY (1<<3) |
| Code change | Changed from (1<<0) to (1<<3) |

## 3.2  MDIO_WRITE and MDIO_READ — Wrong Definitions

| Field | Detail |
| --- | --- |
| Bug | MDIO_WRITE was defined as (1<<1) = 2. This was used as the write trigger, and value 2 was used as the read trigger. |
| Effect | PDF Table 801: 'Read (RD) bit1 — Start a read operation. Write (WR) bit0 — Start a write operation.' The code had READ and WRITE opcodes swapped. mdio_write() was sending READ commands to the PHY and mdio_read() was sending invalid opcodes. PHY CTRL showed 0x0000 after the loopback write because the write never happened. |
| Fix | #define MDIO_READ (1<<1) // bit1 = start read #define MDIO_WRITE (1<<0) // bit0 = start write |
| Code change | Swapped the define values and updated all mdio_read/write calls |

## 3.3  MDIO_PHYSHIFT and MDIO_REGSHIFT — Wrong Bit Positions

| Field | Detail |
| --- | --- |
| Bug | MDIO_PHYSHIFT=10, MDIO_REGSHIFT=5 (taken from greth_test.c original) |
| Effect | PDF Table 801: 'PHYADDR bits[15:11], REGADDR bits[10:6]'. With REGSHIFT=5, register address 2 (PHY_ID1) was placed at bits[9:5] instead of bits[10:6]. GRETH read register 1 (STATUS) instead of register 2 (ID1). Both PHY ID1 and ID2 returned 0x0000. |
| Fix | MDIO_PHYSHIFT=11, MDIO_REGSHIFT=6 (confirmed from greth_api.c: phyaddr<<11, addr<<6) |
| Code change | #define MDIO_PHYSHIFT 11 #define MDIO_REGSHIFT 6 |

## 3.4  MDIO_LINKFAIL — Incorrect Usage in mdio_read

| Field | Detail |
| --- | --- |
| Bug | mdio_read checked if(RD(GRETH_MDIO) & MDIO_LINKFAIL) return 0xDEAD after the busy poll, using the stale value from the busy poll variable rather than a fresh read. |
| Effect | PDF: 'LF reset value = 1'. After GRETH reset, the LF bit is 1 by default. The check caused all MDIO reads to return 0xDEAD immediately after reset, making Test 2 report all IDs as 0xDEAD. |
| Fix | Added one extra load after the post-busy-poll (matching greth_api.c exactly): tmp = RD(GRETH_MDIO); if (tmp & MDIO_LINKFAIL) return 0xDEAD; return (tmp>>16) & 0xFFFF; |
| Code change | Added fresh RD(GRETH_MDIO) after mdio_wait() before checking LF bit |

## 3.5  mdio_read and mdio_write — Missing Pre-Poll

| Field | Detail |
| --- | --- |
| Bug | Both functions polled MDIO_BUSY only AFTER writing the command, not before. |
| Effect | greth_api.c does: poll → write → poll. Without the pre-poll, if a previous MDIO operation was still in progress, the new command would corrupt it. This caused intermittent wrong readback values and the PHY CTRL loopback readback showing 0x0000. |
| Fix | Added mdio_wait() at the start of both mdio_read() and mdio_write() before the WR(GRETH_MDIO,...) call. |
| Code change | Added: if (mdio_wait() < 0) return 0xDEAD; at top of mdio_read() Added: mdio_wait(); at top of mdio_write() |

## 3.6  Test 3 Busy-Poll — Wrong Register and Wrong Bit

| Field | Detail |
| --- | --- |
| Bug | while(GRETH_MDIO & 0x8) — checked the address constant not the register value |
| Effect | GRETH_MDIO is a #define equal to 0x80000D10 (an integer constant). 0x80000D10 & 0x8 = 0 always, so the loop exited immediately without waiting. Also 0x8 = bit3 which is MDIO_BUSY — but the expression evaluated on the address not the register. The PHY loopback write appeared to complete instantly without actually finishing. |
| Fix | while(RD(GRETH_MDIO) & MDIO_BUSY); — reads the register value and checks the correct bit. |
| Code change | while(GRETH_MDIO & 0x8) → while(RD(GRETH_MDIO) & MDIO_BUSY) |

# 4. DMA Descriptor Fixes

## 4.1  RX Descriptor Length Field Should Not Be Pre-Filled

| Field | Detail |
| --- | --- |
| Bug | rxd[i].ctrl = DESC_EN ¦ DESC_WRAP ¦ 128 — pre-filled 128 into the LENGTH field |
| Effect | PDF Table 789: 'LENGTH bits[10:0] — The number of bytes received to this descriptor.' This field is written BY GRETH after reception. Pre-filling it with 128 corrupts the descriptor control word — GRETH may misinterpret the value as status bits. |
| Fix | rxd[i].ctrl = DESC_EN (plus DESC_WRAP on last). No length field. GRETH fills it in after receiving. |
| Code change | rxd[i].ctrl = DESC_EN ¦ DESC_WRAP; // removed ¦ 128 |

## 4.2  Descriptor memset Before Test 5

| Field | Detail |
| --- | --- |
| Bug | Test 5 reused txd and rxd arrays from Test 4 without clearing them. |
| Effect | After Test 4, txd[0] had DESC_WRAP set and old address values. When Test 5 set up new descriptors on top, leftover bits in ctrl words caused GRETH to misread descriptor 0 state and produce data mismatch on frame 0. |
| Fix | Added memset(txd, 0, sizeof(txd)) and memset(rxd, 0, sizeof(rxd)) at the start of test_multi_frame_loopback() before any descriptor setup. |
| Code change | memset(txd, 0, sizeof(txd)); memset(rxd, 0, sizeof(rxd)); |

## 4.3  GRETH_TXDESC/RXDESC Written Before Descriptors Were Ready

| Field | Detail |
| --- | --- |
| Bug | In some versions, GRETH_TXDESC and GRETH_RXDESC were written before descriptor ctrl and addr fields were fully initialized. |
| Effect | PDF: 'care should be taken when writing to the descriptor pointer register. It should never be touched when transmission is active.' Writing the pointer too early or while GRETH is active could leave the internal pointer in a wrong state. |
| Fix | All descriptor fields (addr and ctrl) are fully set up in the for loop BEFORE WR(GRETH_TXDESC,...) and WR(GRETH_RXDESC,...). These pointer writes happen BEFORE WR(GRETH_CTRL, CTRL_TXEN...). |
| Code change | Reordered: fill descriptors → write TXDESC/RXDESC → clear STATUS → write CTRL |

# 5. STATUS Register Handling Fixes

## 5.1  STS_TXIRQ and STS_RXIRQ — Wrong Bit Positions

| Field | Detail |
| --- | --- |
| Bug | STS_TXIRQ = (1<<3), STS_RXIRQ = (1<<2) — these were correct per PDF Table 798. |
| Effect | No issue here — confirmed correct from PDF: bit3=TI, bit2=RI. |
| Status | No change needed — already correct |

## 5.2  Writing STS_ERRORS to STATUS Causes Infinite Loop

| Field | Detail |
| --- | --- |
| Bug | WR(GRETH_STATUS, STS_ERRORS) was used to clear error bits, then the loop continued polling for TXIRQ/RXIRQ. |
| Effect | PDF: 'The transmitter AHB error (TA) bit is set when an AHB error was encountered. Any active transmissions were aborted and the transmitter was disabled.' After an AHB error, GRETH disables itself. Clearing the error bits does not re-enable GRETH. The poll loop then waits for TXIRQ/RXIRQ which never comes — causing an infinite loop lasting TIMEOUT*NUM_FRAMES cycles. |
| Fix | Each error bit is checked individually (STS_TXAHB, STS_RXAHB, STS_TXERR, STS_RXERR). On any error: print which error occurred, clear that specific bit, and break immediately. Do not continue polling. |
| Code change | Replaced STS_ERRORS check with individual if(sts & STS_TXAHB)/if(sts & STS_RXAHB) checks with break |

## 5.3  Stale STATUS Flags Causing False IRQ Triggers

| Field | Detail |
| --- | --- |
| Bug | No WR(GRETH_STATUS, 0xFF) before enabling CTRL in Tests 4 and 5. |
| Effect | IRQ flags left from MDIO activity in Tests 1-3 could immediately trigger the TX/RX poll conditions, causing false pass detection on the very first loop iteration before any frame was actually sent or received. |
| Fix | Added WR(GRETH_STATUS, 0xFF) immediately before WR(GRETH_CTRL, CTRL_TXEN...) in Tests 4 and 5. |
| Code change | WR(GRETH_STATUS, 0xFF); // added before each CTRL enable |

# 6. TX/RX Flow and Multi-Frame Fixes

## 6.1  tx_ok Flag Set Incorrectly

| Field | Detail |
| --- | --- |
| Bug | tx_ok = 1 was set inside if(sts & STS_TXIRQ) — which fires once per frame, not once per all-frames-done. |
| Effect | After frame 0 transmitted, STS_TXIRQ fired, tx_ok was set to 1, and the loop exited. Only 1 of 4 frames was confirmed transmitted. The check 'All TX frames sent' passed incorrectly. |
| Fix | tx_ok = 1 is only set when the inner descriptor loop confirms ALL DESC_EN bits are cleared. STS_TXIRQ is used only to re-enable CTRL_TXEN and clear the IRQ flag — not as the success indicator. |
| Code change | Moved tx_ok=1 to inside the all_done check block, removed from the STS_TXIRQ block |

## 6.2  RE/TE Must Be Re-Asserted After Each IRQ

| Field | Detail |
| --- | --- |
| Bug | CTRL was written once before the loop and not re-written when IRQs fired. |
| Effect | PDF: 'RE should be written with a one each time new descriptors are enabled. As soon as GRETH encounters a disabled descriptor it will stop until RE is set again.' After processing frame 0's descriptor and clearing its EN bit, GRETH checks descriptor 1. If TE/RE is not re-asserted before GRETH reaches the next descriptor, it stops and clears TE/RE automatically. Frames 2 and 3 were never processed. |
| Fix | WR(GRETH_CTRL,...) with TXEN (or RXEN) is called every time STS_TXIRQ (or STS_RXIRQ) fires in the poll loop to keep GRETH scanning. |
| Code change | Added WR(GRETH_CTRL, CTRL_TXEN¦CTRL_FULLD¦CTRL_PROM) inside if(sts & STS_TXIRQ) block |

## 6.3  TX and RX DMA Running Simultaneously — AHB Bus Contention

| Field | Detail |
| --- | --- |
| Bug | CTRL_TXEN¦CTRL_RXEN were set together in Tests 4 and 5, enabling both DMA engines at the same time. |
| Effect | GRETH's TX DMA reads frame data from txbuf over AHB while RX DMA writes received data to rxbuf over AHB. Both share the same AHB master interface. With 4 frames running simultaneously the two engines competed for the bus, resulting in AHB error responses (STS_TXAHB or STS_RXAHB) from frame 2 onwards. Only frame 0 (and sometimes frame 1) completed before the error. |
| Fix | Separate TX and RX into two distinct phases. Phase 1: enable only CTRL_TXEN, wait for all 4 frames to transmit. The loopback frames accumulate in the PHY receive FIFO during this time. Phase 2: enable only CTRL_RXEN, drain all 4 received frames. No AHB contention because only one DMA engine runs at a time. |
| Code change | Phase 1: WR(GRETH_CTRL, CTRL_TXEN¦CTRL_FULLD¦CTRL_PROM); Phase 2: WR(GRETH_CTRL, CTRL_RXEN¦CTRL_FULLD¦CTRL_PROM); |

## 6.4  Re-Enable Correct Engine Only

| Field | Detail |
| --- | --- |
| Bug | Inside the TX loop when TXIRQ fired, the code re-wrote CTRL with both CTRL_TXEN¦CTRL_RXEN. |
| Effect | Re-enabling RXEN during the TX phase started the RX DMA engine prematurely — before all TX frames were sent. This caused the same AHB contention problem as Fix 6.3. |
| Fix | In the TX poll loop, only re-assert CTRL_TXEN. In the RX poll loop, only re-assert CTRL_RXEN. |
| Code change | TX loop: WR(GRETH_CTRL, CTRL_TXEN¦CTRL_FULLD¦CTRL_PROM); RX loop: WR(GRETH_CTRL, CTRL_RXEN¦CTRL_FULLD¦CTRL_PROM); |

# 7. PHY Loopback Fixes

## 7.1  PHY Auto-Negotiation Must Be Disabled for Loopback

| Field | Detail |
| --- | --- |
| Bug | PHY_CTRL_AUTONEG was left enabled (bit12=1) when writing the loopback command. |
| Effect | phy.vhd runs the auto-negotiation counter (anegcnt) on every MDC edge while anegen=1. This continuously overwrites speedsel and duplexmode with auto-negotiated values, fighting against the loopback setting. The loopback echo path became unstable. |
| Fix | Write PHY CTRL as PHY_CTRL_LOOPBACK ¦ PHY_CTRL_100MB ¦ PHY_CTRL_FULLD — without setting bit12 (AUTONEG). Auto-neg disabled = PHY holds the speed/duplex we set. |
| Code change | mdio_write(PHY_ADDR, PHY_CTRL, PHY_CTRL_LOOPBACK ¦ PHY_CTRL_100MB ¦ PHY_CTRL_FULLD); |

# 8. GRETH Reset and Initialization Flow

## 8.1  greth_reset() Does Not Need Complex PHY Init

| Field | Detail |
| --- | --- |
| Bug | An earlier version of greth_reset() attempted a full PHY reset + auto-neg wait sequence, reading back speed and duplex to configure CTRL. |
| Effect | phy.vhd is clocked only on MDC edges, not the system clock. The auto-neg completion poll (status bit5) timed out on the system clock before phy.vhd had enough MDC cycles to finish. Speed and duplex read back as 0 (10Mb half-duplex) causing CTRL to be set incorrectly. |
| Fix | greth_reset() does: WR(GRETH_CTRL, CTRL_RESET) then delay(5000). The delay gives VCS simulation enough time units for phy.vhd to complete its internal MDC-clocked reset and auto-negotiation. CTRL_FULLD is then written directly — no PHY readback needed. |
| Code change | Simplified greth_reset() to: WR(CTRL_RESET) + delay(5000) |

# 9. Complete Change Summary

| # | Category | What Was Wrong | Fix Applied |
| --- | --- | --- | --- |
| 1 | CTRL register | CTRL_PROM = (1<<8) RESERVED bit | Changed to (1<<5) per PDF bit5=PM |
| 2 | CTRL register | CTRL_100MB bit7 written in MII mode — read-only | Removed entirely — not valid in MII mode |
| 3 | CTRL register | CTRL_TXEN¦CTRL_RXEN in Test 1 with no descriptors | Removed — only CTRL_FULLD written in Test 1 |
| 4 | MDIO register | MDIO_BUSY = (1<<0) — wrong bit | Changed to (1<<3) per PDF bit3=BU |
| 5 | MDIO register | MDIO_WRITE = (1<<1) = READ opcode — swapped | MDIO_READ=(1<<1), MDIO_WRITE=(1<<0) per PDF |
| 6 | MDIO register | MDIO_PHYSHIFT=10, MDIO_REGSHIFT=5 — wrong positions | Changed to 11 and 6 per PDF bits[15:11] and [10:6] |
| 7 | MDIO register | MDIO_LINKFAIL check used stale value not fresh read | Added RD(GRETH_MDIO) after busy poll before LF check |
| 8 | MDIO read/write | No pre-poll before writing MDIO command | Added mdio_wait() at start of both functions |
| 9 | Test 3 | while(GRETH_MDIO & 0x8) — address not register, wrong bit | Changed to while(RD(GRETH_MDIO) & MDIO_BUSY) |
| 10 | RX descriptor | rxd.ctrl pre-filled with 128 in LENGTH field | Removed — GRETH fills LENGTH after reception |
| 11 | Test 5 setup | Descriptor arrays not cleared before Test 5 | Added memset(txd,0) and memset(rxd,0) |
| 12 | Test 5 setup | TXDESC/RXDESC written before descriptors were ready | Reordered: fill descs → write ptrs → clear STS → enable CTRL |
| 13 | STATUS handling | WR(STS_ERRORS) after AHB error caused infinite loop | Break immediately on each error bit individually |
| 14 | STATUS handling | No STATUS clear before enabling CTRL | Added WR(GRETH_STATUS, 0xFF) before each CTRL enable |
| 15 | Multi-frame flow | tx_ok set inside TXIRQ handler — only 1 frame confirmed | tx_ok set only when ALL DESC_EN bits are cleared |
| 16 | Multi-frame flow | CTRL_TXEN¦CTRL_RXEN together — AHB bus contention | Separated TX phase (TXEN only) and RX phase (RXEN only) |

# 10. Expected VCS Transcript When All Tests Pass

*****************************************************

*  LEON3 + GRETH Ethernet Verification             *

*  Design : GR-XC3S-1500                           *

*****************************************************

--- TEST 1: GRETH Register Access & MAC Address ---

PASS: GRETH reset bit self-clears

PASS: MAC MSB write-readback

PASS: MAC LSB write-readback

PASS: Full-duplex bit set

--- TEST 2: PHY MDIO Read ---

PHY ID1     = 0xBBCD

PHY ID2     = 0x9C83

PHY STATUS  = 0x786D  (or similar)

PASS: PHY ID1 not 0xFFFF (MDIO responding)

PASS: PHY ID2 not 0xFFFF (MDIO responding)

PASS: PHY STATUS readable

PASS: No MDIO link failure

--- TEST 3: PHY Loopback Mode ---

PHY CTRL after write = 0x6100

PASS: PHY loopback bit confirmed via readback

--- TEST 4: Single Frame Loopback ---

PASS: Single frame TX completed

PASS: No TX errors

PASS: Single frame RX completed

PASS: No RX errors

PASS: Loopback frame payload matches

--- TEST 5: Multi-Frame Stress Loopback (4 frames) ---

txd  @ 0x40013000

rxd  @ 0x40012C00

PASS: All TX frames sent

PASS: No TX errors (stress)

PASS: All RX frames received

PASS: No RX errors (stress)

PASS: All frame payloads match (stress)

*****************************************************

*  RESULTS                                         *

*****************************************************

Passed : 20

Failed : 0

*** GRETH ETHERNET TEST PASSED ***

*****************************************************